Ovrvision Pro firmware

How to Update

Version 1.1

2016/01/21

- Summary

The USB3.0 periphery controller made from Cypress "CYUSB3014-BZXI" is used in Ovrvision Pro. There is a CPU of high-performance ARM9 in which RAM of 512KB was mounted for data reductions, and the arrangement about USB3.0 curb is provided altogether. This document shows fashion to rewrite the Program data in EEPROM which has memorized the firmware which is Program of CPU.

- The firmware rewriting (update) process

First, start the "boot loader" for data rewriting of EEPROM.

- Remove four screws of Ovrvision Pro and open a cover. (PH0 driver)

- Connect USB to your PC, short-circuiting the short area in the board with some metals. Then, the boot loader starts. 
Please use provided that and Reset which it starts where USB is connected.

Be careful not to apply load for crease prevention of USB3.0 connector.

- A WestBridge driver is automatically installed from Windows Update. Please fail and install by provided that in Manual to be installed and "driver" folder -> "WestBridgeDriver.zip."

Write in firmware using downloaded exclusive tools

- Start exclusive tool "ovrvisionprofrm_windows" folder -> "programmer" folder -> "CyControl.exe" program.

- Click and confirm with tool that "Cypress FX3 USB BootLoader Device" has come out.

- Choose "Program" -> "I2C EEPROM". of the MENUBAR

- Because the file selection dialog opens automatically after a while, choose the new firmware "OvrvisionProFrm_v1.img" of the "firmware" folder, and write in EEPROM.

- It will be completion if "Programming of I2C EEPROM in Progress" under a window becomes "Programming of I2C EEPROM in Succeeded."

If set to Faild, please repeat once again.

Finally I cut a hookup for Ovrvision Pro from a personal computer once, and reattach a cover.

- Separate Ovrvision Pro from a personal computer.

- I attach the cover of Ovrvision Pro and tighten four screws.

- It is completion.

- Contact Information

Shionbiya.com Co.,Ltd. Wizapply Team.

KS Bld.5F, 3-7-10, Ichiokamotomachi, Minato-ku Osaka-shi Osaka, 552-0002, Japan

E-Mail :  info@ovrvision.com

Sorry, we will modify later about strange English.

- Revision history

Version 1.0　…　First release

Version 1.1　…　Fixed bugs which transfer stops during camera running.